DRAFT SUPPLY CONTRACT

no: 01-11-6882/26

University Clinical Hospital Mostar, Kralja Tvrtka bb, 88000 Mostar

("The Contracting Authority"),

of the one part,

and

<Full official name and address of the Contractor>

<Official registration number>

<VAT number>, (“the Contractor”)

of the other part,

have agreed as follows:

# CONTRACT TITLE

# The Procurement of Modern Ultrasound System for Cardiovascular Diagnostic

# Identification number 01-11- 6882/26

# Article 1	Subject

# The subject of the contract is the supply, delivery, installation by the Contractor of the following goods:

| Item number. | Specifications Required |  |
| --- | --- | --- |
|  | Premium 4D Cardiovascular Ultrasound System |  |
| 1 | Cart-based ultrasound system with ergonomic front and rear handles |  |
| 2 | Advanced system platform with a software-based beamformer, enabling a variable number of processing channels (minimum 11,400,000) and a variable dynamic range of at least 380 dB |  |
| 3 | 4 active transducer ports, excluding pencil probe connectors. |  |
| 4 | HDU LED monitor, minimum 23.8" diagonal size, mounted on a movable articulated arm, with a minimum resolution of 1920 × 1080 pixels. |  |
| 5 | Integrated ultra-high-resolution LCD touch screen panel, minimum 12.1" diagonal size. |  |
| 6 | Operator control panel with an integrated or software-based alphanumeric keyboard and interactive backlighting, adjustable in height and depth, with the possibility of rotation. |  |
| 7 | Maximum B-mode scanning depth of at least 50 cm, depending on the transducer |  |
| 8 | Maximum 2D mode frame rate of at least 2500 fps, depending on the transducer, system settings, and application |  |
| 9 | Maximum volume rate in 4D mode with a 4D transthoracic transducer of at least 800 volumes per second (vps). |  |
| 10 | Maximum volume rate in 4D mode with a 4D transesophageal transducer of at least 540 volumes per second (vps). |  |
| 11 | Left ventricular opacification (LVO) using contrast agents. |  |
| 12 | Advanced contrast imaging software with low mechanical index (MI) values |  |
| 13 | Automatic optimization of image quality in 2D mode |  |
| 14 | Automatic spectral optimization technique activated by a single button press |  |
| 15 | Real-time triplex imaging with continuous-wave Doppler: 2D imaging + Color Flow Mapping (CFM) + Continuous Wave Doppler (CW). |  |
| 16 | Advanced real-time image noise reduction technology based on pixel-level grayscale analysis. |  |
| 17 | Real-time spatial compounding technology for combining images acquired from different angles into a single enhanced image. |  |
| 18 | Automatic cardiac Doppler measurement function |  |
| 19 | The system supports operation with the following types of transducers: |  |
|  | 19.1 | Convex transducers |
|  | 19.2 | Linear transducers |
|  | 19.3 | Phased array transducers |
|  | 19.4 | Matrix phased array transducers |
|  | 19.5 | TEE transducers (Transesophageal Echocardiography transducers) |
|  | 19.6 | 4D TEE matrix transducers |
|  | 19.7 | Matrix linear transducers |
| 20 | Operating modes: |  |
|  | 20.1 | 2D Tissue Mod (B-Mode) |
|  | 20.2 | M-Mode |
|  | 20.3 | Anatomical M-mode |
|  | 20.4 | Curved anatomical M-mode |
|  | 20.5 | 2D Color Flow Mode (CFM) |
|  | 20.6 | 2D Angio Flow (Power Doppler) |
|  | 20.7 | PW Doppler |
|  | 20.8 | TVI, TVD, Tissue Tracking |
|  | 20.9 | CWD |
|  | 20.10 | 4D Tissue |
|  | 20.11 | 4D Color Flow |
| 21 | Qualitative assessment of global and segmental myocardial deformation by combining three longitudinal views, providing a comprehensive evaluation displayed as a bull’s-eye diagram. |  |
| 22 | Automated one-click myocardial deformation analysis. An artificial intelligence (AI)-based algorithm for automatic region of interest (ROI) generation enables complete analysis workflow, from tool initiation to validation of the obtained results, without requiring any user interaction. |  |
| 23 | Automatic ejection fraction measurement based on speckle tracking algorithm. |  |
| 24 | Automated one-click ejection fraction measurement. An artificial intelligence (AI)-based algorithm for automatic region of interest (ROI) generation enables ejection fraction measurement on recordings acquired with or without ECG, without requiring any user interaction from tool activation to validation of the obtained results. |  |
| 25 | Tool for quantitative assessment of right ventricular global longitudinal strain (RV GLS), right ventricular free wall strain, and segmental strain based on the apical four-chamber (A4C) view, with automatic measurement of TAPSE. |  |
| 26 | Tool for quantitative assessment of left atrial global longitudinal strain (LA GLS), left atrial volume, and left atrial emptying fraction (LAEF). |  |
| 27 | Stress echocardiography package supporting 2D pharmacological stress, 2D bicycle exercise, and 2D treadmill exercise protocols, including dedicated protocols for the assessment of global and segmental longitudinal myocardial strain at all stages of stress testing. |  |
| 28 | Dedicated software for left ventricular quantification based on 4D echocardiographic acquisition. |  |
| 29 | Simultaneous visualization in dual- and tri-plane B-mode imaging, with or without Color Flow Mapping (CFM) or Tissue Velocity Imaging (TVI). |  |
| 30 | Simultaneous visualization of at least 12 combined standard planes/views extracted from 4D echocardiographic acquisition, available in real-time (live) mode or during post-processing. |  |
| 31 | Advanced 3D visualization technique for enhanced depth perception of volumetric objects, utilizing sophisticated shading and reflection algorithms combined with depth rendering technologies, allowing placement of the light source behind the rendered tissue to create a backlighting effect. The functionality shall be available for both live imaging and stored/reviewed images. |  |
| 32 | Dedicated tool allowing placement of markers and annotations within 4D echocardiographic acquisitions. |  |
| 33 | Digital imaging technique enabling real-time visualization of vascular hemodynamics by direct visualization of blood reflectors and grayscale representation of hemodynamic information. |  |
| 34 | Artificial intelligence (AI)-based automated cardiac 2D measurements on the parasternal long-axis (PLAX) view. |  |
| 35 | Artificial intelligence (AI)-based function that automatically recognizes the type of spectral Doppler trace and automatically initiates the corresponding Doppler measurements. |  |
| 36 | DICOM 3.0 |  |
| 37 | Min. 2 USB 3.0 porta |  |
| 38 | DVI-D and DisplayPort video outputs. |  |
| 39 | Ethernet |  |
| 40 | Integrated 3-lead ECG module. |  |
| 41 | Sonde: |  |
|  | 39.1 | Transthoracic 4D matrix array sector transducer, manufactured using single-crystal technology, with a frequency range of 1.5–3.8 MHz or wider, a minimum field of view (FOV) of 90°, and a minimum of 2,300 elements. |
|  | 39.2 | 2D linear transducer with a frequency range of 3–9 MHz or wider, a minimum field of view (FOV) of 38 mm, and a minimum of 190 elements. |
| 42 | ### Available hardware and software upgrade options for future system expansion and enhancement. |  |
|  | 40.1 | ### Fusion of 4D ultrasound acquisition with CT acquisition in DICOM format. |
|  | 40.2 | ### 4D transesophageal matrix array transducer with a frequency bandwidth of 3.5–6.8 MHz or wider, providing a minimum field of view (FOV) of 90° and featuring at least 2,300 elements. |
| 43 | Integrated black-and-white thermal printer. |  |
| 44 | Maximum system weight: 121 kg (±10% tolerance). |  |

# Note: The tenderer shall submit a declaration, signed and stamped by an authorised representative, confirming that all software modules, applications and functionalities offered and specified in the Technical Specifications are perpetually licensed and fully enabled, and that their use is not time-limited nor subject to the payment of additional licence fees, subscriptions, activation fees or any other subsequent charges for the use of the functionalities covered by this procurement throughout the operational lifetime of the equipment.

The place of acceptance of the supplies shall be At University Clinical Hospital Mostar, , Clinical Department of Cardiology, , Bijeli Brijeg bb, 88000 Mostar, the time limits for supply, delivery and installation shall be 60calendar days from the signing of the Contract and the Incoterm applicable shall be DDP.

Period of implementation of tasks is 60 calendar days in total from contract signature. This period includes supply, delivery and installment of the equipment in 30 calendar days by the tenderer and 30 days for Contracting Authority to issue certificate of provisional acceptance.

1.2	The Contractor shall comply strictly with the terms of the Special Conditions and the technical annex.

# Article 2	Origin

The rules of origin of the goods are defined in Article 10 of the Special Conditions.

# Article 3	Price

3.1	The price of the supplies shall be that shown on the financial offer (specimen in Annex IV). The total maximum contract price shall be in BAM.

3.2 	Payments shall be made in accordance with the General and Special Conditions (Articles 26 to 28).

# Article 4	Order of precedence of contract documents

The contract is made up of the following documents, in order of precedence:

- the contract agreement;

- the Special Conditions

- the General Conditions (Annex I);

- the Technical Specifications (Annex II [including clarifications before the deadline for submission of tenders and minutes from the information meeting/site visit];

- the Technical Offer (Annex III [including clarifications from the tenderer provided during tender evaluation]);

- the budget breakdown (Annex IV);

- Financial identification Form (Annex V)

- Legal entity Form (Annex V)

- other relevant documents (Annex V)

- Performance guarantee

# The various documents making up the contract shall be deemed to be mutually explanatory; in cases of ambiguity or divergence, they shall prevail in the order in which they appear above.

Done in English in four originals, two original/s being for the Contracting Authority-Project Partner, and two originals being for the Contractor.

| For the Contractor |  | For the Contracting Authority-Project Partner |  |
| --- | --- | --- | --- |
| Name: |  | Name: |  |
| Title: |  | Title: |  |
| Signature: |  | Signature: |  |
| Date: |  | Date: |  |

<DDP (Delivered Duty Paid)>/<DAP (Delivered At Place)> - Incoterms 2020 International Chamber of Commerce - https://iccwbo.org/business-solutions/incoterms-rules/incoterms-2020/